SEMINOLE

PA44-180

BEFORE ENGINE START

seat belts/door	SECURE

gear selector 	DOWN

cross-feed	CHECK

flight controls	CHECK

all electrical	OFF

circuit breakers	IN

baggage	SECURE

passengers	BRIEF

brake	SET

throttles 	CLOSED

mixtures 	IDLE

props 	FORWARD

carb heat 	OFF

cowl flaps 	OPEN

START ENGINE NORMAL

fuel valves	ON

master switch	ON

alternators	2 ON

gear lights	GREEN

throttle 	CRACK

mixture 	FORWARD

props	FORWARD

boost pumps	2 ON

beacon	ON

magnetos	ALL ON

brakes	HOLD

propeller	CLEAR!

primer/starter	ENGAGE

throttle	1000rpm

oil pressure	GREEN

ammeter	CHECK

suction	ALIVE

radios	ON

transponder	STBY

boost pumps 	OFF

flaps	UP

mixture	LEAN 1”

gyros 	SLAVE

lights 	AS REQ.

autopilot 	TRIM off/test

electric trim 	CHECK/SET

ATIS	OBTAIN

clearance	OBTAIN

crossfeed 	X-feed

area	CLEAR

brakes	TEST

INSTRUMENTS	TAXI CHECK

RUN UP

brakes	HOLD

flight controls	CHECK

trim	SET

fuel valve	ON

mixture	RICH

throttles	1500 rpm

props 	FEATHER chk

throttles 	2000 rpm

magnetos	L/R chk

175 mx, 50 df

props	CYCLE 3x

300 drop max

suction	4.8 to 5.2

oil pres/temp	GREEN

carb heat	CHECK

alternator	CHECK

annunciators 	chk/OUT

throttle 	IDLE chk

throttle	1000 rpm

navaids	SET

altimeter	SET

heading ind.	SET

radios	GRD  TWR

area	CLEAR

INSTRUMENTS	TAXI CHECK

BEFORE TAKEOFF

INSTRUMENTS	CHECK

cowl flaps 	OPEN

fuel valves	ON

crossfeed	OFF

flaps	SET

mixtures	RICH

props	FORWARD

carb heat	OFF

trim	SET

radios	TWR / DEP

fuel pumps	ON

BLT or lights/camera/action

ENGINE SHUT DOWN

brakes	HOLD

heater 	COOL 2 min, off

fuel	CALL (130.80)

radios/nav	OFF

transponder	OFF

elec switches	OFF

beacon	OFF

mixture/mags/master

SEMINOLE

Flight

BEFORE TAKEOFF

INSTRUMENTS	CHECK

cowl flaps 	OPEN

fuel valves	ON

crossfeed	OFF

flaps	SET

mixtures	RICH

props	FORWARD

carb heat	OFF

trim	SET

radios	TWR / DEP

fuel pumps	ON

window	CLOSED

lights	ON

transponder 	ALT

CLIMB

PUFF:

gear/flaps	UP

cowl flaps 	CLOSED

speed	BLUE LINE

1000 AGL

MP/props	25"/2500

boost pumps	OFF

manifold	RAISE w/alt

mixture	LEAN

CRUISE

boost pump	OFF

landing light	OFF

cowl flaps 	CLOSED

power	24"/2200

(table)

mixture	LEAN

heading ind.	SET

fuel/oil/suc/amm	CHECK

DESCENT

ATIS	obtain

altimeter	set

heading ind.	SET

mixture	rich

carb heat	as needed

power	as needed

APPROACH SETUP

boost pump 	ON

mixture 	RICH

power 	20”/2400

gear	DOWN blw 140

carb heat 	AS REQ.

BEFORE LANDING

GUMPS	GUMPS

cowl flaps 	AS REQ.

autopilot 	OFF

flaps	AS NEEDED

landing light	ON

APPROACH: 15, Full, Flaps, 95

MP	15”-17”

props	FORWARD

GO AROUND

PUFF

mix/props/pwr	FULL

carb heat	OFF

climb	POSITIVE

flaps - gear - flaps - flaps

CLEARED OFF RUNWAY

flaps	UP

carb heat	OFF

cowl flaps 	OPEN

fuel pumps	OFF

transponder	OFF

landing light	OFF

mixture	LEAN 1 INCH

radios	GRD

taxi clearance	OBTAIN

pitot/wind heat	OFF

flight plan	CLOSE

INST APPROACH	Six T’s

ATIS	Turn

Altimeter	Time

Approach	Twist

Avionics	Throttle

Airspeed	Talk

Alignment	Track

SEMINOLE

PROBLEMS

ENGINE FAILURE AFTER LIFTOFF, no runway left

Mix/props/MP	FORWARD

flaps	UP

landing gear	UP

airspeed	BLUE LINE

IDENTIFY-VERIFY-FEATHER

bank	3º, 1/2 ball

climb	VYSE = 88

trim	3º rudder, VYSE

SECURE INOP

cowl flap 	CLOSE

alternator	OFF

mags	OFF

fuel pump	OFF

fuel valve	OFF

RETURN FOR	LANDING

SINGLE ENGINE LANDING

GOOD ENGINE:

Boost pump	ON

Carb heat	AS NEEDED

Gas	ON

(Crossfeed	OFF)

Undercarriage	AUTO/HAND

Mixture	RICH

Prop	FORWARD

Cowl flap	AS REQ

Seat belt	SECURE

airspeed       95 until landing sure

altitude           HIGH until ldg sure

flaps               when landing sure

CROSSFEED ENGAGE

fuel select, good engine   X-feed

fuel select, bad engine     OFF

boost fuel pumps	            OFF

fuel pressure		    CHECK

CROSSFEED DISENGAGE

fuel select, good engine    ON

fuel select, bad engine      OFF

boost pumps                      OFF

fuel pressure                      CHK

ENGINE OUT

ENGINE OUT

1. mixture	RICH

2. props	FORWARD

3. manifold	FULL

4. flaps	UP

5. landing gear	UP

airspeed	BLUE LINE

bank	3º, 1/2 ball

IF 3000 AGL:

boost pumps	ON

carb heat	ON

fuel	CHECK

oil/fuel gauges	NORMAL

mags	ON

ID-VERIFY-FEATHER

identify	DEAD FOOT

verify	THROTTLE

feather	PROP BACK

mixture	IDLE CUT OFF

SECURE

cowl flap	CLOSED

alternator 	OFF

mags	OFF

fuel pump	OFF

fuel select	OFF

electric load	CHECK/REDUCE

crossfeed	SET

power	25/25 good engine

LAND	ASAP

UNFEATHERING aka RESTART

fuel selects 	BOTH ON

mags	ON

fuel pump	ON

prop	FORWARD

mixture	RICH

(prime	throttle 2 strokes)

throttle	OPEN 1/4"

(speed	INCREASE)

or

(starter	ENGAGE)

warm engine	10" few minutes

alternator	ON

fuel pump	OFF

power	UP

Seminole

EMERGENCY

BOTH ENGINES OUT

airspeed	MAINTAIN

props	FEATHER

mixtures	CUTOFF

fuel valves	OFF

mags	OFF

squawk	7700

radio	MAYDAY

seatbelts	ON

gear	DOWN

master	OFF

doors	OPEN

EMERGENCY DESCENT

throttles	RETARD

props	FORWARD

airspeed	VNO /VA (rough)

bank	60º

ICING

pitot heat	ON

deicing equip	ON

alternate static	AS NEEDED

heat/defrost	MAX

heading	RETREAT?

altitude	CHANGE

power	INCREASE

land	FASTER usual

SPIN RECOVERY

throttles	IDLE

rudder	FULL OPP.

yoke	NEUTRAL

ailerons	NEUTRAL

attitude	RECOVER

MANUAL LANDING GEAR

CHECK:

nav lights	OFF

circuit breakers	CHECK

master/alternators	ON

EXTEND:

airspeed	  below 100 kts gear selector	  DOWN

emer. gear ext knob  PULL

gear lights	3 GREEN

emer. gear ext knob     leave out

FIRE

ENGINE FIRE IN FLIGHT

fuel selector	OFF

throttle 	IDLE

prop	FEATHER

mixture	IDLE CUTOFF

cowl flap 	OPEN

fuel pump	OFF

power	24/24 good engine

flaps/gear	UP

alternator	OFF

mags	OFF

electric load	REDUCE

IF FIRE PERSISTS

attempt to blow out      DIVE

engine	SECURE

LAND	ASAP

ENGINE FIRE ON GROUND

fuel valve	OFF

fuel pump	OFF

throttle	MAX

mixture	IDLE CUTOFF

other engine	SHUT DOWN

EVACUATE, EXTINGUISH

ELECTRICAL FIRE

master switch	OFF

alternators 	OFF

elec switches	OFF

radio master 	OFF

vents	CLOSE

cabin heat 	OFF

circuit breakers	CHECK, PULL

fire extinguisher	USE

WHEN FIRE OUT

master	ON

vents	OPEN

switches & CBs	ONE AT A TIME

CABIN FIRE

vents	CLOSE

fire extinguisher	USE

WHEN FIRE OUT

cabin	VENTILATE

SEMINOLE

PREFLIGHT

weather	CHECKED

flight plan	FILED

documents	A.R.O.W

baggage	SECURE

weight/balance	CHECK

charts/nav	ABOARD

performance	COMPUTED

hobbs	RECORDED

control wheel 	RESTRAINTS

windshield	CLEAN

static system	DRAIN

alternate static	NORMAL

mags	OFF

parking brake	SET

boost pumps	OFF

gear selector	DOWN

throttles	IDLE

mixtures	IDLE CUT-OFF

cowl flaps	OPEN

flight controls	CHECK

trims	NEUTRAL

fuel selectors	ON

all electrical	OFF

master	ON

beacon	CHECK

fuel gauges	CHECK

annunciators      PRESS/CHECK

flaps	LOWER

gear lights	3 GREEN

pitot heat	CHECK

VOR	CHECK

master	OFF

RIGHT WING:

fuel sumps	SAMPLE

surface	CHECK

flap	CHECK

aileron	CHECK

wing tip/lights	CHECK

scupper drain	CLEAR

fuel vent	CLEAR

tie down	REMOVE

fuel and cap	CHECK

oil	6+ QUARTS

prop/spinner	CHECK

air inlets	CLEAR

generator belt	TIGHT

cowl flap area	CHECK

oleo strut	~ 2.6"

tire	CHECK

brake     DISK/PADS/LEAKS

gear well/door	CHECK

chocks	REMOVE

NOSE:

battery vents	CLEAR

landing lights	CHECK

heater air inlet	CHECK

gear well/door	CHECK

oleo strut	~ 2.7"

tire	CHECK

chocks	REMOVE

LEFT WING:

surface	CHECK

oleo strut	~ 2.6"

tire	CHECK

brake     DISK/PADS/LEAKS

gear well/door	CHECK

chocks	REMOVE

cowl flap area	CHECK

fuel and cap	CHECK

oil	6+ QUARTS

prop/spinner	CHECK

air inlets	CLEAR

generator belt	TIGHT

scupper drain	CLEAR

fuel vent	CLEAR

tie down	REMOVE

stall warning	CHECK

pitot/static mast	CHECK

wing tip/lights	CHECK

aileron	CHECK

flap	CHECK

LEFT FUSELAGE:

skin/belly	CHECK

emergency exit 	CHECK

antennas	SECURE

air inlet 	CHECK

EMPENNAGE:

surface	CHECK

elevator/rudder	CHECK

lights	SECURE

tie down	REMOVE

RIGHT FUSELAGE:

skin	CHECK

baggage door	SECURE

Seminole quick reference

KIAS

VYSE	=	  88 (blue line)

VY	=	  88

VX	=	  82

VMC	=	  56 (red line)

VA	=	112 to 135

VS0	=	  55 (white arc)

VS1	=	  57 (green arc)

VNO	=	169 (green arc)

VNE	=	202 (yellow arc)

VFE	=	111 (white arc)

VLE	=	140

VLR	=	109

VXSE	=	  82

VSSE	=	  82

VR	=	  75

ENGINES

180 hp

air-cooled

2700 RPM max

Feather lock below 950 rpm

WEIGHT

852ND Basic empty: 2602.92

Ramp max: 3816

Takeoff/landing max: 3800

Baggage max: 200

Useful load: 1197

Full fuel payload: ~ 550

FUEL AND OIL

Fuel: 108 usable

Worst fuel use: ~ 12

gal/hr. each

Oil: 6-8 per engine

Heater: gasoline combustion,

uses ½ gallon per hour from the left tank

LANDING GEAR

hydraulic, pump-operated

Gear warning 14” MP

or 2 notches flaps

ELECTRICAL

12V, 35 ampere-hour battery

Two 14V 60-amp alternators

WEIGHT AND BALANCE for 852ND:

|  | Weight (lbs) | Arm | Moment (in-lbs) |
| --- | --- | --- | --- |
| Basic empty weight | 2602.9 | 85.88 | 223538 |
| Front seats |  | 80.5 |  |
| Rear seats |  | 118.1 |  |
| Fuel |  | 95.0 |  |
| Baggage |  | 142.8 |  |
| Ramp wt (3816 max) |  |  |  |
| TOTALS |  |  |  |

CG limits:

Wt	forward	rearward

2800	84.0	93.0

3400	85.0	93.0

3800	89.0	93.0

Max-performance takeoff

Wt	rotate	Vx climb

3800	70	82

3400	66	77

3000	62	72

Accelerate-stop distance [(takeoff + landing)*2] ~ 3800 ft.

Max crosswind: 17 knots